| MODULE CODE |  | MODULE TITLE |  |
| --- | --- | --- | --- |
| MMME3085 |  | Computer Engineering and Mechatronics |  |
| ASSESSMENT TITLE |  |  |  |
| Final Project Submission – Drawing Robot Software |  |  |  |
| ASSESSMENT TYPE |  |  | WEIGHT (INDICATIVE EFFORT) |
| Coursework |  |  | 30% |
| DEADLINE DATE |  | DEADLINE TIME | SUBMISSION METHOD |
| Wednesday 10th of December |  | 15:00 | moodle submission box |
| STAFF MEMBER RESPONSIBLE |  |  |  |
| Giovanna Martínez Arellano |  |  |  |
| FEEDBACK DETAILS |  |  |  |
| A mark, and comments on moodle/uploaded file will be provided by |  |  |  |
| LEARNING OUTCOMES ASSESSED |  |  |  |
| LO(X)- | Learning outcomes relevant to the assessment only (Include AHEP mapping)) |  |  |
| KEY SUBMISSION REQUIREMENTS |  |  |  |
| - Git repo to be submitted containing: VS Code, documentation and updated flowcharts - The final document should contain: - an explanation of precisely what the program needs to do - the file structure, particularly if you separated your code in multiple files. - function prototypes (final coded version) - data items you intend to use and their types (final coded data items) - test data |  |  |  |
| NO EVIDENCE EC ELIGIBLE? |  | EC OUTCOME |  |
| No |  | NA |  |
| Before submitting an EC, please discuss with your tutor. |  |  |  |
| SUPPORT PLAN EXTENSION |  |  |  |
| If you have an existing support plan and require a support plan extension for this assessment, you should email EZ-M3-DLO@exmail.nottingham.ac.uk. Please include module code, assessment name, original deadline including time, and requested extension (usually one working week). |  |  |  |
| MISCONDUCT INFORMATION |  |  |  |
| The university considers the unauthorised use of AI tools in assessments to constitute false authorship, which is an academic offence under the Academic Misconduct Policy (section 2.2.2)3. For this assignment, AI may be used. However, you need to declare how you have used it. This means you will add a section on the final project documentation that explains how you use it, for which element of the report, provide the inputs that you have given the AI and also include the outputs given by the AI tool. |  |  |  |

# Computer Engineering and Mechatronics Project: Developing software for a drawing robot

# Part 2: Final Project Submission

## Introduction

Full details of the project were supplied in the project planning document, Computer Engineering and Mechatronics Project Planning. For the remainder of the project you should develop the software as planned in the project planning assignment.

## Testing of your application

A basic simulator is available at  https://nraynaud.github.io/webgcode/ to allow you to verify the commands intended to be sent to the drawing robot. Information on how to configure the project files to print G-codes to use with the simulator are given in Appendix 1 (note that this is the same as Appendix 5 in the planning document).

Make sure that you test your code with different text and sizes of font. Note that a new ‘Test.txt’ file will be provided at the robot testing.

Testing of your code with the real robot will take place on Tuesday 2nd, Wednesday 3rd and Friday 5th December in UP ESLC B01. A spreadsheet will be circulated closer to that date to allow booking of a time slot.

The rest of this document explains what is required for the final project submission.

## Submission

Software Project Final Submission (30%)

Learning outcomes:

- To write robust code for a mechatronics problem using a specification (as developed in the project planning submission)

- To develop the documentation required to allow the project to be developed by a team

- To be able to use version control for software development

Learning outcomes will be demonstrated in the submission documents by:

- Developing the code to fulfil the project brief, which is well structured and follows software engineering good practice (e.g. well named variables, error trapping).

- Providing a system manual which could be used by a software development team to maintain or continue to develop the code including the following:

  - A clear explanation of what the program does, including sample emulator output (which should show your custom shape).

  - Description of files used in the program

    - Describe all .c and .h files used in the project.

    - Indicate where configuration changes may be needed during development.

  - Description of functions used in the program including:

    - Function declarations (prototypes)

    - A brief description (one or two lines) of what the function does

    - Parameter definition and their types

    - Show whether parameters are input and/or output, whether they are changed and the return value, if any.

    - Include functions which you have written yourself, not the ones which are supplied in the program template.

  - Specification of the main data items used in the program.

    - Give data type and why this has been chosen.

  - Test data which will validate the program, confirming conformance of the program/function to its specification.

    - Ensure that all routes through the program are covered.

    - Include input data with expected outputs.

  - Flowcharts which show the structure of the program

    - Include blocks to indicate function calls and separate flowcharts for functions where appropriate

  - Using the git repository set up at the planning stage of the project

    - Updates to code during the development process	should be committed regularly

    - Planning documents should be updated where necessary and changes committed to the repository

Note: If your code development followed your project planning accurately then the system manual will be largely the same as your project planning submission, with the addition of the section describing the files used in the project. If, in the process of developing your code, you found that you needed to diverge from your plan then you should update the system manual and flowcharts to reflect your final code.

A template is provided for the system manual on Moodle (SystemManualTemplate25-26.docx). Save this as SystemManualXXX_ID.docx  where XXX are your initials and ID is your student ID. The flowchart(s) may be submitted in a separate pdf saved as SystemFlowchartXXX_ID.pdf (do not submit a drawio file, use the export function in draw.io to save to pdf).

You will be required to upload a single zip file named SoftwareProjectXXX_ID.zip containing your repository folder.

Your repository should contain:

- Your VSCode project folder which should include:

  - All project files

  - Any .txt files used to test your program

- System manual

- Flowcharts

The deadline for submission to Moodle of this document is 15:00 Wednesday 10th December.

# Appendix 1: Using the skeleton program to develop and test code

The skeleton code described in Appendix 4 of the planning document can also be run without connecting the Arduino, in order to test the code output. In this case a set of dummy functions are run which print the G-codes created to the terminal. These can then be cut and pasted into the G-code simulator (https://nraynaud.github.io/webgcode/) to check the commands created.

The following sections describe the use of compiler preprocessor formatting directives to achieve this. Note that when using #define directives in VSCode (in both Windows and Mac), the section of code to be compiled will be shown in normal text while any code which is not compiled, due to the conditional formatting, will be greyed.

## Using the simulator and writing robot in Windows

Use the RobotWriter2025_Skeleton project. Use the skeleton code as follows to output the demonstration G-codes in the skeleton program and then to develop your own project code:

The preprocessor directive #define Serial_Mode is defined at the top of the serial.c file. When this is defined the code in the top half of the file, following the #ifdef Serial_Mode directive, will be compiled. This will execute the code for the virtual RS232 using the Arduino as described in Appendix 4. Use this condition for the robot testing.

To print G-codes for use with the simulator, comment the #define instruction                    (i.e. //#define Serial_Mode). The code in the second half of the file, following the #else instruction will now be compiled. When the RS232 library commands, such as PrintBuffer or WaitForDollar, are called from main.c they will now execute the ‘dummy’ functions which either print text to the terminal, or wait for a key press to emulate the signal being returned from the serial port.

Note that commenting and uncommenting the #define command is all that is needed to switch between the two modes. The code written to create G-codes in main.c is exactly the same for both modes and no extra code needs to be written to allow for testing.

Robot testing:

Uncomment the #define Serial_Mode line and recompile the code. It will now be ready for testing with the robot.

## Using the simulator with a Mac computer and setup for robot testing

Use the RobotWriter2025_SkeletonMac project. The skeleton code can only be used in emulator mode on a Mac computer as the RS232 library has not been developed for use on Macs.

Use the skeleton code as follows to output the demonstration G-codes in the skeleton program and then to develop your own project code:

Two preprocessor directives are used in this skeleton program: #define MAC is found at the top of both main.c and serial.c and #define Serial_Mode is at the top of serial.c.

When #define MAC is defined the file unistd.h will be included giving the correct library for compilation on a Mac.

In order to execute the emulator code, comment the #define Serial_Mode directive (i.e. //#define Serial_Mode)in serial.c. The code in the second half of the file, following the #else instruction, will be compiled. When the RS232 library commands, such as PrintBuffer or WaitForDollar, are called from main.c they will now execute the ‘dummy’ functions which either print text to the terminal, or wait for a key press to emulate the signal being returned from the serial port.

Robot testing:

- Comment the #define MAC instruction in both main.c and serial.c. The RS232 and Windows headers will now be included.

- Uncomment the #define Serial_Mode line in serial.c. The RS232 functions for communication via the Arduino will now be compiled.

- Copy your project to the Windows computer which will be available in the lab and compile in VSCode.

- Run your robot test!

EC regulations: https://www.nottingham.ac.uk/qualitymanual/assessment-awards-and-deg-classification/ext-circumstances.aspx

Support plans: https://www.nottingham.ac.uk/studentservices/servicedetails/disability-support-services/disability-support-services.aspx

Details of the University’s policy procedures on Academic Misconduct and disciplinary procedures: https://www.nottingham.ac.uk/qualitymanual/assessment-awards-and-deg-classification/pol-academic-misconduct.aspx